Parking Monitor

User Manual – Moving the Whole System to Another PC

ArUco-marker parking spot monitor with an A9 / V720 WiFi camera, a phone app (via Tailscale) and push notifications (ntfy), running 24/7 on a Windows PC.

Covers: what to copy, installing everything on the new PC, camera WiFi, phone app, notifications, 24/7 auto-start, final checks and troubleshooting.

System version: October 2026 (monitor with AP-mode camera, Tailscale app, ntfy notifications, Windows Task Scheduler).

Contents

1. How the system works	4

1.1 Parts of the system	4

1.2 The data path	4

2. Before you start – what you need	5

2.1 Hardware	5

2.2 Accounts and apps	5

2.3 Your current settings (write these down)	5

3. Step 1 – On the OLD PC: stop and copy	6

3.1 Stop the background monitor	6

3.2 Copy the project	6

3.3 Retire the old PC (after the new PC works)	6

4. Step 2 – On the NEW PC: install software	7

4.1 Python 3.11	7

4.2 Tailscale	7

4.3 WiFi adapter driver	7

5. Step 3 – Copy the project and set up Python	8

6. Step 4 – Connect the camera WiFi	9

6.1 Connect	9

6.2 Check the adapter name	9

6.3 Two one-time settings (PowerShell as administrator)	9

7. Step 5 – Test the camera	10

7.1 Quick camera test	10

7.2 Live view and marker check	10

8. Step 6 – Check the settings file	11

9. Step 7 – Phone app (Tailscale)	12

9.1 Publish the app (new PC)	12

9.2 Install the app on the phone	12

9.3 Keep it working on the phone (Android)	12

9.4 Notifications (ntfy)	12

10. Step 8 – Run 24/7	13

10.1 Install the auto-start task	13

10.2 Never sleep when plugged in	13

10.3 Laptop lid	13

10.4 Sign in automatically after a restart	13

11. Step 9 – Final checks	14

11.1 Health check	14

11.2 Restart test	14

12. Everyday use	15

12.1 Notifications you can receive	15

13. Troubleshooting	16

14. Appendix – files in the project	17

Quick command card	17

# 1. How the system works

Each parking spot has a printed ArUco marker on the ground. The camera looks down at the spots. If a marker is visible, the spot is FREE; if a car covers it, the spot is OCCUPIED. A reference marker (ID 9) sits where no car can cover it: if the camera cannot see it (dark, lens covered, camera moved), the system says camera blind and does not change any spot.

## 1.1 Parts of the system

| Part | What it does | Where |
| --- | --- | --- |
| A9 / V720 camera | Streams video over its own WiFi network Nax_5c002000E0F3 (camera address 192.168.169.1, port 6123). Must be on USB power. | Above the board, looking down |
| USB WiFi adapter (TP-Link Archer TX1U Nano) | Connects the PC to the camera WiFi (shows as Wi-Fi 2). | USB port of the PC |
| PC built-in WiFi | Connects the PC to the internet (shows as Wi-Fi). | PC |
| Parking monitor (Python) | Reads the video, detects markers, decides FREE / OCCUPIED, serves the app page, sends notifications. | parking-monitor folder |
| a9-v720 library | Talks the camera's protocol. Must sit next to the parking-monitor folder. | a9-v720 folder |
| Tailscale | Private, encrypted link so your phone can open the app from anywhere. | PC + phone |
| ntfy | Push notifications to the phone when a spot or the camera changes. | Phone app + ntfy.sh |
| Task Scheduler task "ParkingMonitor" | Starts the monitor automatically at sign-in and restarts it if it crashes. | Windows |

## 1.2 The data path

Camera   --(Nax WiFi)----------> USB adapter (Wi-Fi 2)  --> PC

Internet <--(FOREFRONT WiFi)---- built-in Wi-Fi         <-- PC

PC: parking monitor -> app page on 127.0.0.1:8080 (private)

Phone (anywhere)   --(Tailscale)--> https://<pc-name>.<tailnet>.ts.net

Phone (app closed) <-- ntfy push notifications

# 2. Before you start – what you need

## 2.1 Hardware

☐  New Windows 10 / 11 PC or laptop that can stay on and plugged in.

☐  The USB WiFi adapter (move it from the old PC). Any adapter with 2.4 GHz support works.

☐  The camera with its USB power cable and a USB charger or free USB port.

☐  The camera stand and the board with the printed markers (IDs 1, 2 and 9).

☐  A USB stick or cloud drive to copy the project folder.

## 2.2 Accounts and apps

☐  Your Tailscale account (the one used on the phone).

☐  The ntfy app on the phone, subscribed to your topic (no change needed).

☐  Internet on the new PC (FOREFRONT or any WiFi / Ethernet).

## 2.3 Your current settings (write these down)

| Setting | Current value | Changes on new PC? |
| --- | --- | --- |
| Camera WiFi name | Nax_5c002000E0F3 | No |
| Camera address / port | 192.168.169.1 / 6123 | No |
| Camera firmware | 202509111709 | No |
| Spot markers | ID 1 = Spot A, ID 2 = Spot B | No |
| Reference marker | ID 9 | No |
| Marker size | about 2.3–2.5 cm black square | No |
| Camera height | about 16 cm, looking straight down | No |
| ntfy topic | parking-0ea239f45303 | No (keep the same) |
| App address (Tailscale) | https://laptop-t1iis3tf.tail163fff.ts.net | Yes – the new PC gets a new address |

Important: The ntfy topic works like a password for your notifications. Keep it private.

# 3. Step 1 – On the OLD PC: stop and copy

## 3.1 Stop the background monitor

Open PowerShell (Start → type PowerShell) and run:

Stop-ScheduledTask ParkingMonitor

## 3.2 Copy the project

Copy the whole Parking Monitor folder from the Desktop to a USB stick. It must contain both folders side by side:

Parking Monitor\

parking-monitor\   <- the monitor (config, code, markers, scripts)

a9-v720\           <- camera library (must stay next to parking-monitor)

You can skip these sub-folders inside parking-monitor (they are rebuilt or not needed):

| Folder | Why skip it |
| --- | --- |
| .venv | Python environment – must be re-created on the new PC (it does not work when copied). |
| logs | Old log files. |
| status | Old status file – recreated automatically. |
| ap_test_out | Old camera test results and snapshots (may contain private pictures). |

Tip: Keep the folder path short on the new PC (e.g. Desktop or C:\ParkingMonitor). Very long paths can break Python installs on Windows.

## 3.3 Retire the old PC (after the new PC works)

Do this only when the new PC is running correctly (Section 11), so you always have one working system:

- Remove auto-start: Unregister-ScheduledTask ParkingMonitor -Confirm:$false

- Turn off the app link: tailscale serve --https=443 off

- Optional: remove the old PC from your account at login.tailscale.com → Machines.

- Optional: undo automatic sign-in (Section 10.4, "To undo").

# 4. Step 2 – On the NEW PC: install software

## 4.1 Python 3.11

- Download Python 3.11 (64-bit) from python.org → Downloads → Windows.

- Run the installer. On the first screen tick "Add python.exe to PATH", then click Install Now.

- Check it in PowerShell: python --version should print Python 3.11.x.

Note: Python 3.11 is the version this system was built and tested with. Newer versions usually work, but 3.11 is the safe choice.

## 4.2 Tailscale

- Download Tailscale for Windows from tailscale.com/download/windows and install it.

- Click the Tailscale icon in the taskbar (near the clock, maybe under the ^ arrow) → Log in with the same account as your phone.

## 4.3 WiFi adapter driver

- Plug in the USB WiFi adapter.

- Usually Windows installs it by itself within a minute. If a TP-LINK drive appears in File Explorer instead, either run SetupInstall.exe from it, or (safer) download the official driver from tp-link.com → Support → Archer TX1U Nano.

- When it works, a second WiFi adapter appears in the taskbar WiFi menu.

# 5. Step 3 – Copy the project and set up Python

- Copy the Parking Monitor folder from the USB stick to the new PC's Desktop.

- Open PowerShell in the project folder: in File Explorer open Parking Monitor\parking-monitor, click the address bar, type powershell, press Enter.

- Create the Python environment and install the libraries (takes 1–3 minutes):

python -m venv .venv

.venv\Scripts\pip install -r requirements.txt

Then run the automatic tests. It should end with "27 passed":

.venv\Scripts\python -m pytest -q

Important: Install only opencv-contrib-python (from requirements.txt). Do not also install opencv-python – the two conflict and marker detection stops working.

# 6. Step 4 – Connect the camera WiFi

The PC needs two WiFi connections at the same time: the USB adapter on the camera, the built-in WiFi on the internet.

## 6.1 Connect

- Plug the camera into USB power and wait about 1 minute.

- Click the WiFi icon in the taskbar. Choose the USB adapter (usually called Wi-Fi 2) and connect it to Nax_5c002000E0F3. Tick Connect automatically.

- Make sure the built-in WiFi (Wi-Fi) is connected to your internet WiFi.

## 6.2 Check the adapter name

In PowerShell run the command below. Note which name belongs to the TP-Link adapter (usually Wi-Fi 2). If it has another name, use that name in the next commands.

Get-NetAdapter

## 6.3 Two one-time settings (PowerShell as administrator)

Open Start → type PowerShell → right-click → Run as administrator. Paste each line and press Enter:

a) Reconnect to the camera automatically after a restart:

netsh wlan set profileparameter name="Nax_5c002000E0F3" interface="Wi-Fi 2" connectionmode=auto

b) Always use the built-in WiFi for internet (otherwise Windows may try to reach the internet through the camera, and the app and notifications stop working):

Set-NetIPInterface -InterfaceAlias "Wi-Fi 2" -InterfaceMetric 500

Tip: If PowerShell shows >> instead of running a command, press Ctrl + C and paste the command again as a single line.

# 7. Step 5 – Test the camera

## 7.1 Quick camera test

In parking-monitor, double-click run_ap_test.bat. After about 2 minutes it writes ap_test_out\report.txt. Look for:

- Camera reachable on port 6123

- streamed 40 s ... size 640x480 (video works)

Note: If the camera is not reachable: check its USB power, check Wi-Fi 2 is connected to Nax_5c002000E0F3, then run the test again.

## 7.2 Live view and marker check

- Double-click run_monitor.bat. A camera window and a black console window open.

- Each marker should get a green outline with a label like id1 65px. All three (id1, id2, id9) must be seen, each 40 px or more (25 minimum).

- The top line should say camera: ok  REF OK  free: 2/2.

- Park a toy car on a spot: after about 3 seconds it turns occupied. Remove it: free again.

- Press q in the camera window to stop.

| Problem | Fix |
| --- | --- |
| A marker has no green outline | Move it fully into view (not at the image edge), flatten it, add light, remove glare. |
| Size below 40 px | Lower the camera a little. |
| Picture blurry | Raise the camera a little until sharp. |
| REF MISSING / camera blind | Marker 9 is not fully visible – move the board or turn the camera. |

# 8. Step 6 – Check the settings file

Open parking-monitor\config.yaml with Notepad. Most values stay the same. Check these:

| Setting | Value | Meaning |
| --- | --- | --- |
| source: type | a9v720 | Use the A9 camera. |
| a9v720: mode | ap | PC connects to the camera's own WiFi. |
| spots | 1: "Spot A", 2: "Spot B" | Marker ID → spot name. Add lines for more spots. |
| reference_marker_id | 9 | Marker that must always be visible. |
| confirm_seconds | 3 | Seconds a change must last (toy cars: 3, real cars: 10). |
| camera_alert_seconds | 30 | Seconds before a camera blind / offline notification. |
| notifications: topic | parking-0ea239f45303 | Your ntfy topic – keep the same. |
| notifications: click_url | your app address | Update to the new PC's Tailscale address (Section 9). |
| web: bind | 127.0.0.1 | App page private to the PC – do not change. |

Important: YAML is sensitive to spaces. Keep the same indentation (2 spaces) and do not use Tab.

### Adding more parking spots

- Add a line under spots, e.g. 3: "Spot C" (any ID 0–49 except 9).

- Make the markers: .venv\Scripts\python parking_monitor.py gen --size-cm 2.5 --dpi 300 --paper a4

- Print markers\markers.pdf at 100 % / Actual size on matte paper.

# 9. Step 7 – Phone app (Tailscale)

The app address contains the PC name, so the new PC gets a new address. Set it up once:

## 9.1 Publish the app (new PC)

- Make sure the monitor is running (Section 10.1 installs the auto-start; for now you can double-click run_monitor.bat).

- In PowerShell run:

& "C:\Program Files\Tailscale\tailscale.exe" serve --bg 8080

- If it says "Serve is not enabled on your tailnet", open the link it shows, click Enable (approve HTTPS), then run the command again.

- It prints your new app address, e.g. https://new-pc-name.tailxxxx.ts.net. Write it down.

- Put this address in config.yaml → notifications: click_url, so tapping a notification opens the app.

## 9.2 Install the app on the phone

- Open the Tailscale app on the phone and switch it ON (key icon in the status bar).

- Delete the old Parking icon from the home screen (it points to the old PC).

- Open the new address in the browser and add it to the home screen:

  - Android: Chrome → ⋮ menu → Install app (or Add to Home screen).

  - iPhone: Safari → Share → Add to Home Screen.

## 9.3 Keep it working on the phone (Android)

- Settings → Apps → Tailscale → Battery saver → No restrictions.

- Optional: Settings → Connection & sharing → VPN → Tailscale → Always-on VPN.

Note: Only devices signed in to your Tailscale account can open the app. It is not public on the internet and not visible to other people on the WiFi.

## 9.4 Notifications (ntfy)

Nothing to change on the phone – the topic stays the same. Test from the new PC:

.venv\Scripts\python parking_monitor.py test-notify

A notification "Test from parking monitor" should arrive within seconds. If not: check the topic in the ntfy app is exactly parking-0ea239f45303, and set ntfy to No restrictions in the phone's battery settings.

# 10. Step 8 – Run 24/7

## 10.1 Install the auto-start task

Close any run_monitor.bat window first (press q). Then in PowerShell, in the parking-monitor folder:

powershell -ExecutionPolicy Bypass -File deploy\windows\install_task.ps1

It prints "Installed and started". The monitor now runs in the background (no window), starts at every sign-in and restarts itself if it crashes.

## 10.2 Never sleep when plugged in

- Settings → System → Power & battery → Screen and sleep → "When plugged in, put my device to sleep after" → Never. (The screen may turn off – that is fine.)

## 10.3 Laptop lid

- Control Panel → Power Options → Choose what closing the lid does → Plugged in: Do nothing → Save changes.

- Optional (ASUS): MyASUS → Battery Health Charging → Balanced (80 %) to protect the battery.

## 10.4 Sign in automatically after a restart

The monitor starts when Windows is signed in. To sign in without a click after a power cut or Windows Update:

### If the account has NO password (only a "Sign in" button)

Run these three lines in PowerShell as administrator. Replace User with the new PC's account name if different (check with whoami):

reg add "HKLM\SOFTWARE\Microsoft\Windows NT\CurrentVersion\Winlogon" /v AutoAdminLogon /t REG_SZ /d 1 /f

reg add "HKLM\SOFTWARE\Microsoft\Windows NT\CurrentVersion\Winlogon" /v DefaultUserName /t REG_SZ /d User /f

reg add "HKLM\SOFTWARE\Microsoft\Windows NT\CurrentVersion\Winlogon" /v DefaultPassword /t REG_SZ /d "" /f

### If the account HAS a password

- Press Windows + R, type netplwiz, press Enter.

- Untick "Users must enter a user name and password" → OK → type the password twice.

- If the tickbox is missing: Settings → Accounts → Sign-in options → turn off "only allow Windows Hello sign-in", then try again.

Note: To undo automatic sign-in: run the first reg command with /d 0 instead of /d 1. With automatic sign-in, anyone with physical access to the PC gets in – fine for a PC that stays at home.

# 11. Step 9 – Final checks

## 11.1 Health check

In the parking-monitor folder run (it only looks, it changes nothing):

powershell -ExecutionPolicy Bypass -File tools\check_network.ps1

All lines should be [OK]:

| Check | If it fails |
| --- | --- |
| Two WiFi adapters | Plug in / install the USB WiFi adapter (Section 4.3). |
| One adapter on the camera WiFi | Connect Wi-Fi 2 to Nax_5c002000E0F3 (Section 6). |
| Camera reachable | Camera USB power on? Wi-Fi 2 connected? |
| Internet reachable | Run the metric command (Section 6.3 b). |
| App page running | Install the task (Section 10.1) or start it: Start-ScheduledTask ParkingMonitor. |
| Tailscale installed / serving | Section 4.2 and 9.1. |
| Startup task installed | Section 10.1. |
| PC never sleeps on charger | Section 10.2. |

## 11.2 Restart test

- Restart the PC and do not touch it.

- It should reach the desktop by itself (no Sign in click).

- After 1–2 minutes open the Parking app on the phone (Tailscale on): spots shown and Camera OK.

- Park a toy car: a notification arrives within a few seconds; tapping it opens the app.

If all of this works, the move is complete. You can now retire the old PC (Section 3.3).

# 12. Everyday use

| I want to… | Do this |
| --- | --- |
| Check the parking | Open the Parking app on the phone (Tailscale on). Works from anywhere. |
| See the live camera | In the app tap Show camera (uses about 50–70 MB per hour of mobile data while shown). |
| See the debug window on the PC | Double-click run_monitor.bat. It pauses the background monitor and restarts it when you close the window. |
| Check the system | powershell -ExecutionPolicy Bypass -File tools\check_network.ps1 |
| Stop the monitor | Stop-ScheduledTask ParkingMonitor |
| Start the monitor | Start-ScheduledTask ParkingMonitor |
| See what happened | Open parking-monitor\logs\parking.log in Notepad (newest lines at the bottom). |
| Send a test notification | .venv\Scripts\python parking_monitor.py test-notify |
| Remove auto-start | Unregister-ScheduledTask ParkingMonitor -Confirm:$false |
| Turn off the app link | tailscale serve --https=443 off |

## 12.1 Notifications you can receive

| Notification | Meaning |
| --- | --- |
| Spot A is FREE / Spot A is taken | A spot changed and stayed changed for confirm_seconds. |
| Parking camera blind | Reference marker 9 not visible for 30 s (dark, covered, camera moved). Spots are frozen until it is visible again. |
| Parking camera offline | No video for 30 s (camera off / no power / Wi-Fi 2 disconnected). |
| Parking camera OK | The camera recovered. |

Important: Only one monitor can use the camera at a time. Do not start a second copy; run_monitor.bat handles this for you by pausing the background monitor.

# 13. Troubleshooting

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| App: "Can't reach the parking PC" / site can't be reached | Tailscale off on the phone, or PC off / asleep, or monitor stopped | Switch Tailscale ON on the phone; check the PC is on; run the health check; Start-ScheduledTask ParkingMonitor. |
| App: "Camera offline" | Camera has no power, or Wi-Fi 2 disconnected | Plug the camera into USB power; reconnect Wi-Fi 2 to Nax_5c002000E0F3. The monitor reconnects by itself within about 15 s. |
| App: "Camera can't see clearly" (blind) | Marker 9 not fully visible, too dark, camera moved | Put marker 9 fully in view; add light; re-aim the camera (check with run_monitor.bat). |
| Spot stays FREE with a car on it | Car does not cover enough of the marker | Move the marker to where the car body stops; use a slightly smaller marker. |
| Spot flickers | Camera shaking, poor light, marker too small | Fix the camera firmly; even light; marker at least 40 px. |
| No notifications | Phone battery saver, wrong topic, no internet on the PC | ntfy → No restrictions; check topic; run the health check (Internet reachable). |
| Internet stops working on the PC | Windows uses the camera WiFi for internet | Run the metric command (Section 6.3 b). |
| Camera window: nothing / "offline" after restart | Wi-Fi 2 not set to connect automatically | Run the auto-connect command (Section 6.3 a). |
| "Another parking monitor is already running" | Background monitor already uses the camera | Normal protection. Use run_monitor.bat (it pauses the background one) or Stop-ScheduledTask ParkingMonitor first. |
| pip install fails / very long path error | Project folder path too long | Move the Parking Monitor folder to the Desktop or C:\ParkingMonitor and try again. |
| Phone on camera WiFi cannot open the page | The camera WiFi blocks phone-to-PC traffic | Expected – use the Tailscale app address instead. |

# 14. Appendix – files in the project

| File / folder | Purpose |
| --- | --- |
| parking_monitor.py | Main program: run, gen (make markers), probe, test-notify. |
| config.yaml | All settings (camera, spots, timings, app, notifications). |
| requirements.txt | Python libraries to install. |
| run_monitor.bat | Live debug window (pauses / restarts the background monitor). |
| run_ap_test.bat | Camera connection test – writes ap_test_out\report.txt. |
| run_preview.bat | App demo on a sample video (no camera needed). |
| parking\ | Program code: sources (camera), detector (markers), state (free/occupied logic), monitor (main loop), web (phone app), output (status file + notifications), markers (printing). |
| tests\ | Automatic tests (python -m pytest -q). |
| tools\check_network.ps1 | Health check (changes nothing). |
| tools\ap_test.py | The camera test used by run_ap_test.bat. |
| deploy\windows\install_task.ps1 | Installs the 24/7 auto-start task. |
| markers\, markers_2.5cm\ | Printable markers (print at 100 %). |
| logs\parking.log | Log file (rotates automatically, max about 6 MB). |
| status\parking_status.json | Current status file (also served to the app). |
| ..\a9-v720\src\ | Camera protocol library (third-party, github.com/intx82/a9-v720). |

## Quick command card

# in the parking-monitor folder

python -m venv .venv

.venv\Scripts\pip install -r requirements.txt

.venv\Scripts\python -m pytest -q

powershell -ExecutionPolicy Bypass -File deploy\windows\install_task.ps1

powershell -ExecutionPolicy Bypass -File tools\check_network.ps1

& "C:\Program Files\Tailscale\tailscale.exe" serve --bg 8080

.venv\Scripts\python parking_monitor.py test-notify

Stop-ScheduledTask ParkingMonitor   /   Start-ScheduledTask ParkingMonitor